FY B.Tech Computer Engineering

2020 Course

Computational Science

VISHWAKARMA INSTITUTE OF INFORMATION TECHNOLOGY, PUNE

COMPUTER ENGINEERING DEPARTMENT

BY:

NAME: Prajwal Waykos

G.R NO.: 22010591

ROLL NO.: 4036

CLASS: FY

BATCH: D2

# EXPERIMENT NO. 7

Aim: Spectrophotometric or Colorimetric Estimation of Fe+3 from a given solution.

:

- Determination of λmax

Concentration of solution = 0.005 mg/ml

| Filter No. |  | Absorbance (A) |
| --- | --- | --- |
| 1. | 400 | 0.03 |
| 2. | 420 | 0.14 |
| 3. | 470 | 0.32 |
| 4. | 500 | 0.35 |
| 5. | 530 | 0.30 |
| 6. | 620 | 0.05 |
| 7. | 660 | 0.03 |
| 8. | 700 | 0.01 |

Graph I: (1 Mark+2Marks))

From graph I ,  λmax = 	500	nm

- Determination of Unknown Concentration (2 Marks)

| Sr. No. | Standard Fe solution | Concentration in mg/ml | (% T) | A = - Log T |
| --- | --- | --- | --- | --- |
| 1. | 5 ml | 0.001 | 77 | 0.11 |
| 2. | 10 ml | 0.002 | 74 | 0.13 |
| 3. | 15 ml | 0.003 | 63 | 0.20 |
| 4. | 20 ml | 0.004 | 54 | 0.27 |
| 5. | 25 ml | 0.005 | 46 | 0.34 |
| 6. | Blank | - | (100) | 0 |
| 7. | Unknown | ? | 73 | 0.14 |

# Calculations:

Concentration of standard Fe solution = 0.01 mg/ml Using C1 V1 = C2 V2, calculate concentration of all solutions.

1.  0.01 * 5 = C2 *50

C2 = 0.001 mg/ml

2. 0.01 * 10 = C2 * 50

C2 = 0.002 mg/ml

3. 0.01 * 15 = C2 * 50

C2 = 0.003 mg/ml

4. 0.01 * 20 = C2 * 50

C2 = 0.004 mg/ml

5. 0.01 * 25 = C2 * 50

C2 = 0.005 mg/ml

Plot a graph of absorbance vs. concentration

Graph 2: (2Marks)

Unknown Concentration

From the standard curve obtained, determine the concentration of given unknown sample.

Results: (2 Marks)

1) λmax = 	500 	nm.

2) Concentration of iron present in a given sample= 	0.0021 	mg/ml.

Questions (1mark)

Q.1) State Lambert’s law and Beer’s law.

Ans) Lambert’s Law: When a beam of monochromatic light is allowed to pass through a transparent medium, the rate of decrease of radiant power with the thickness of the medium is directly proportional to the thickness of medium or the path length.

Beer’s Law: When a beam of monochromatic light is allowed to pass through a transparent medium, the rate of decrease of radiant power with the concentration of the medium is directly proportional to the radiant power i.e., absorbance of the solution is directly proportional to the concentration of the solution.